Verilog code for top module Soc

module SoC (

input clk,                // System clock

input reset,              // System reset

output [7:0] led_out      // LED output (8-bit)

);

// Wires to connect Timer peripheral

wire [7:0] timer_count;

wire timer_interrupt;

// Internal memory (Single-Port RAM: 255 x 8-bit)

reg [7:0] memory [0:254];

// LED output register

reg [7:0] led_reg;

assign led_out = led_reg;

// Timer Instance

Timer timer_inst (

.clk(clk),

.reset(reset),

.count(timer_count),

.interrupt(timer_interrupt)

);

// State machine for interrupt response

reg [1:0] state;

reg [15:0] delay_counter; // simple delay counter

localparam IDLE     = 2'b00,

ON_STATE = 2'b01,

DELAY    = 2'b10,

OFF_STATE= 2'b11;

always @(posedge clk or posedge reset) begin

if (reset) begin

led_reg <= 8'h00;

memory[0] <= 8'h00;

state <= IDLE;

delay_counter <= 16'd0;

end else begin

case (state)

IDLE: begin

if (timer_interrupt) begin

led_reg <= 8'hFF;       // Turn on LEDs

memory[0] <= 8'h55;     // Write 0x55 to memory[0]

delay_counter <= 16'd50000; // Some delay cycles

state <= DELAY;

end

end

DELAY: begin

if (delay_counter > 0)

delay_counter <= delay_counter - 1;

else

state <= OFF_STATE;

end

OFF_STATE: begin

led_reg <= 8'h00;         // Turn off LEDs

memory[0] <= 8'h00;       // Clear memory[0]

state <= IDLE;

end

endcase

end

end

endmodule

module Timer (

input clk,                 // Clock input

input reset,               // Reset input

output reg [7:0] count,    // 8-bit count value (from 0F to 00)

output reg interrupt       // Interrupt signal when count hits 00

);

always @(posedge clk or posedge reset) begin

if (reset) begin

count <= 8'h0F;        // Initialize counter to 0F on reset

interrupt <= 1'b0;

end else begin

if (count == 8'h00) begin

interrupt <= 1'b1; // Raise interrupt when count reaches 00

end else begin

count <= count - 1'b1;  // Decrement count

interrupt <= 1'b0;      // Clear interrupt if not at 00

end

end

end

endmodule

Verilog code for testbench

`timescale 1ns / 1ps

module SoC_tb;

// Testbench signals

reg clk;

reg reset;

wire [7:0] led_out;

// Instantiate the SoC

SoC uut (

.clk(clk),

.reset(reset),

.led_out(led_out)

);

// Clock generation: 10ns period (100 MHz)

initial begin

clk = 0;

forever #5 clk = ~clk;

end

// Stimulus block

initial begin

// Monitor output

$monitor("Time: %0t | Reset: %b | LED: %h", $time, reset, led_out);

// Initialize

reset = 1;

#20;

// Deassert reset

reset = 0;

// Run simulation for some time to observe behavior

#500000;

// Finish simulation

$finish;

end

endmodule

Assembly code

AREA    RESET, CODE, READONLY

ENTRY

LDR     R0, =0x40000000      ; LED GPIO

LDR     R1, =0x20000000      ; RAM base

LDR     R2, =0x00000010      ; Timer ISR vector

; Infinite loop

main_loop

B       main_loop

AREA    ISR, CODE, READONLY

EXPORT  Timer_Handler

Timer_Handler

; Write FF to LED

LDR     R3, =0xFF

STRB    R3, [R0]

; Write 55 to RAM[0]

LDR     R3, =0x55

STRB    R3, [R1]

; Simple delay

MOV     R4, #0xFF

delay_loop

SUBS    R4, R4, #1

BNE     delay_loop

; Clear LED

MOV     R3, #0x00

STRB    R3, [R0]

; Clear RAM[0]

STRB    R3, [R1]

; Clear timer interrupt flag if needed

BX      LR

Results